SAFETY DATA SHEET

Compiled in accordance with REACH Regulation (EC) No 1907/2006, as amended by UK REACH Regulations SI 2019/758

SECTION 1: Identification of the Substance/Mixture and of the Company/Undertaking

1.1. Product Identifier

Zoflora Concentrated Disinfectant (All Perfumes) – 9405771

1.2. Relevant identified uses of substance or mixture and uses advised against

Household disinfectant.

1.3. Details of supplier of the safety data sheet

Supplier name:		Thornton & Ross Ltd

Supplier address:		Linthwaite, Huddersfield, HD7 5QH

Telephone:		+44 (0) 1484 842 217

Fax:			+44 (0) 1484 847 301

Email:			sds@thorntonross.com

1.4. Emergency telephone number

Telephone:		+44 (0) 1484 848 164

Available:		Out of working hours

SECTION 2: Hazards Identification

2.1. Classification of the substance or mixture

According to Regulation (EC) No. 1272/2008 (GB-CLP) the mixture is classified as Flammable Liquid Category 2, Skin Irritant Category 2, Eye Irritant Category 2 and Hazardous to Aquatic Environment – Chronic Hazard Category 3.

2.2. Label elements

Pictogram(s):

Signal Word:			Danger

Hazard Classification:		Flammable Liquid Category 2

Skin Irritant Category 2

Eye Irritant Category 2

Hazardous to Aquatic Environment Chronic Category 3

Hazard Statements:		H225: Highly flammable liquid and vapour

H315: Causes skin irritation

H319: Causes serious eye irritation

H412: Harmful to aquatic life with long lasting effects

Precautionary Statements:		P101: If medical advice is needed, have product container or label at hand.

P102: Keep out of reach of children.

P210: Keep away from heat, hot surfaces, sparks, open flames and other ignition sources, no smoking.

P280: Wear protective gloves/eye protection.

P305+351+338: IN IN EYES – rinse continuously with water for several minutes, remove contact lenses if present and easy to do so – continue rinsing.

P337+313: Get medical advice/attention.

P501: Dispose of contents by pouring down drain with plenty of water, empty containers can be disposed of by recycling where possible, contact your local council for details.

EUH Statements:	EUH208: Contains (see section 2.2.1).  May produce an allergic reaction.

2.2.1 Supplemental Hazard Information Relating to Perfumes

Blackcurrant & Jasmine:	Contains 4-tert-butylcyclohexyl acetate, linalool, linalyl acetate and hydroxycitronellol.  May produce an allergic reaction.

Bluebell Woods:	Contains linalool, benzyl salicylate, hexylcinnamaldehyde, limonene, citronellol, and lilial, May produce an allergic reaction.

Bouquet:	Contains nopyl acetate, hexylcinnamaldehyde, geraniol, cedryl methyl ether, linalyl acetate and linalool. May produce an allergic reaction.

Coastal Waves:	Contains linalyl acetate, linalool, limonene, tetramethyl acetyloctahydronaphthalenes,  citronellol and d-damascone. May produce an allergic reaction.

Coconut and Lime:	Contains hexylcinnamaldehyde, linalool and d-damascone. May produce an allergic reaction.

Country Garden:	Contains benzyl salicylate, tetramethyl acetyloctahydronaphthalenes and citronellol. May produce an allergic reaction.

Cranberry and Orange:	Contains d-limonene, eugenol, tetrahydrolinalool, coumarin and cinnamaldehyde. May produce an allergic reaction.

Cucumber & Mint	Contains hexylcinnamaldehyde, linalool, geraniol, tetramethyl acetyloctahydronaphthalenes, citronellol, menthone, geranyl acetate and hexyl salicylate. May produce an allergic reaction.

Cypress and Sea Sage:	Contains linalool, limonene, and benzyl salicylate. May produce an allergic reaction.

Fig & Cedar:	Contains linalool, hexylcinnamaldehyde, linalyl acetate, tetramethyl acetyloctahydronaphthalenes, citronellol and cinnamaldehyde. May produce an allergic reaction.

Fir Needle & Amber:	Contains limonene and methyl cedryl ketone, eugenol and cinnamaldehyde. May produce an allergic reaction.

Frosted Peppermint:	Contains linalool and trans-menthone. May produce an allergic reaction.

Honeysuckle and Jasmine:	Contains hexylcinnamaldehyde, citronellol, benzyl salicylate, limonene, linalool, butylbenzyl propionaldehyde and geraniol. May produce an allergic reaction.

Hyacinth Bloom:	Contains linalool, hexylcinnamaldehyde, benzyl salicylate, citronellol, cyclamen aldehyde and limonene. May produce an allergic reaction.

Lavender:	Contains linalyl acetate, cineole, dihydromyrcene, methyl cedryl ketone, linalool. May produce an allergic reaction.

Lemon Zing:	Contains linalool, citral and geraniol. May produce an allergic reaction.

Linen Fresh:	Contains linalool, benzyl salicylate, tetramethyl acetyloctahydronaphthalenes and linalyl acetate. May produce an allergic reaction.

Mandarin & Lime:	Contains linalool, limonene, hexylcinnamaldehyde and linalyl acetate. May produce an allergic reaction.

Midnight Blooms:	Contains linalool, longifolene and benzyl salicylate. May produce an allergic reaction.

Mountain Air:	Contains dihydromyrcene, linalool, hexylcinnamaldehyde, cineole and limonene. May cause an allergic reaction.

Peony Blush:	Contains hexylcinnamaldehyde, citronellol, linalool, d-lemonene, geraniol and  d-damascone. May produce an allergic reaction.

Pink Grapefruit:	Contains linalool, limonene and hexylcinnamaldehyde. May produce an allergic reaction.

Raspberry & Juniper Berry:	Contains linalool, tetramethyl acetyloctahydronaphthalenes, hexylcinnamaldehyde, citronellol, tetrahydrolinalool and d-damascone. May cause an allergic reaction.

Rhubarb & Cassis:	Contains 4-tert-butylcyclohexyl acetate, benzyl salicylate, linalyl acetate, hexyl salicylate, limonene and bis(cyclohexylmethyl)ether. May cause an allergic reaction.

Rose Noir:	Contains hexylcinnamaldehyde, linalool, 4-tert-butylcyclohexyl acetate, citronellol, tetramethyl acetyloctahydronaphthalenes and d-damascone. May produce an allergic reaction.

Springtime Fresh:	Contains linalyl acetate, linalool, hexyl cinnamaldehyde, amyl cinnamaldehyde, citronellol, geraniol and cyclamen aldehyde. May produce an allergic reaction.

Summer Berries:	Contains linalool. May produce an allergic reaction.

Summer Breeze:	Contains hexylcinnamaldehyde, geraniol, citronellol and linalool. May produce an allergic reaction.

Sunshine Escapes:	Contains hexylcinnamaldehyde, benzyl salicylate, d-limonene, linalool, 4-tert-butylcyclohexyl acetate, linalyl acetate, tetramethyl acetyloctahydronaphthalenes and citronellol. May produce an allergic reaction.

Sweet Orchard:	Contains linalool, hexylcinnamaldehyde, geranyl acetate, limonene, geraniol and cyclamen aldehyde. May produce an allergic reaction.

Sweet Pea:	Contains geraniol, linalool, dihydromyrcene, citronellol, nerol, and benzyl salicylate. May produce an allergic reaction.

Tropical Palms:	Contains linalool and d-limonene. May cause an allergic reaction.

Vanilla & Amber:	Contains linalool, linalyl acetate, hexylcinnamaldehyde, methyl cedryl ketone, limonene, coumarin. May produce an allergic reaction.

Velvet Poppy:	Contains linalool, citronellol, geraniol, linalyl acetate and d-damascone. May produce an allergic reaction.

Winter Morning:	Contains, linalool, alpha-methylcinnamaldehyde, eugenol, isolongifolanone, cineole and cinnamaldehyde. May produce an allergic reaction.

Winter Spice:	Contains d-limonene and eugenol. May produce an allergic reaction.

2.3. Other hazards

None known

SECTION 3: Composition/Information on Ingredients

3.2. Mixtures

| Hazardous Ingredient(s) | %w/w | CAS no EC/List no REACH reg. no | Hazard Classification | H Statements |
| --- | --- | --- | --- | --- |
| Zoflora Base |  |  |  |  |
| Ethanol 9002380 | 55-65% | 64-17-5 200-578-6 01-2119457610-43-xxxx | Flam Liq 2 | 225 |
| Benzalkonium chloride 9002397 | <2% | 68424-85-1 270-325-2 01-2119983287-23-xxxx | Ac Tox 4, Sk Cor 1B, Eye Dam 1, Aq Ac 1, Aq Chr 1 | 302, 314, 318, 400, 410 |
| Methanol (denaturant for Ethanol) 9002380 | <3% | 67-56-1 200-659-6 01-2119433307-44-xxxx | Flam Liq 2, Ac Tox 3, STOT SE 1 | 225, 301, 311, 331, 370 |
| Zoflora Perfume Compounds |  |  |  |  |
| Perfume compounds (see below) | 1-15 | N/A |  |  |
| Blackcurrant & Jasmine 6030881 no further production, still selling through finished stock |  |  | Sk Sens 1, Eye Irr 2, Aq Chr 3 | 317 319 412 |
| Bluebell Woods 5506664 international only fragrance, under review for new mod |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Bouquet 6099674 |  |  | Eye Irr 2, Sk Sens 1, Aq Chr 2 | 319, 317, 411 |
| Coastal Waves 5914814 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Coconut and Lime 5613562 |  |  | Sk Sens 1, Aq Acu 1, Aq Chr 2 | 317, 400, 411 |
| Country Garden 6064542 |  |  | Sk Sens 1, Aq Chr 2 | 317, 411 |
| Cranberry & Orange 6142871 |  |  | Sk Sens 1, Aq Chr 2 | 317, 411 |
| Cucumber & Mint 6030551 no further production, still selling through finished stock |  |  | Sk Irr 2, Sk Sens 1, Eye Dam 1, Aq Chr 2 | 315, 317, 318, 411 |
| Cypress & Sea Sage 6030611 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Fig & Cedar 5980698 under review for discontinuation |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Fir Needle & Amber 6044705 no further production, still selling through finished stock |  |  | Sk Sens 1, Aq Chr 2 | 317, 411 |
| Frosted Peppermint 5914833 no further production, still selling through finished stock |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 3 | 315, 319, 317, 412 |
| Honeysuckle & Jasmine 5521878 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 317, 319, 411 |
| Hyacinth Bloom 5943665 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Lavender 6099678 |  |  | Sk Irr 2, Eye Dam 1, Sk Sens 1, Aq Chr 2 | 315, 318, 317, 411 |
| Lemon Zing 5599692 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Linen Fresh 6012184 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 3 | 315, 319, 317, 412 |
| Mandarin & Lime 5907978 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Midnight Blooms 6153909 |  |  | Eye Irr 2, Sk Sens 1, Aq Chr 2 | 319, 317, 411 |
| Mountain Air 6064577 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Peony Blush 6156457 new mod code progressing |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Pink Grapefruit 5944766 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Raspberry & Juniper Berry 6030550 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Rhubarb & Cassis 6030612 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Rose Noir 5913038 international only fragrance, under review for new mod |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Springtime Fresh 6031046 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Summer Berries 5676253 |  |  | Sk Sens 1 | 317 |
| Summer Breeze 5370749 new mod required for international |  |  | Sk Irr 2, Eye Dam 1, Sk Sens 1, Aq Chr 2 | 315, 318, 317, 411 |
| Sunshine Escape 6098406 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 3 | 315, 319, 317,412 |
| Sweet Orchard 5912619 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Sweet Pea 6048755 |  |  | Sk Irr 2, Eye Dam 1, Sk Sens 1, Aq Chr 2 | 315, 318, 317, 411 |
| Tropical Palms 6129350 |  |  | Sk Irr 2, Eye Dam 1, Sk Sens 1, Aq Chr 2 | 315, 318, 317, 411 |
| Vanilla & Amber 6002582 |  |  | Sk Irr 2, Eye Irr 2, Sk Sens 1, Aq Chr 2 | 315, 319, 317, 411 |
| Velvet Poppy 6031062 no further production, still selling through finished stock |  |  | Sk Irr 2, Eye Dam 1, Sk Sens 1, Aq Chr 2 | 315, 318, 317, 411 |
| Winter Morning 6143592 |  |  | Sk Sens 1, Aq Chr 3 | 317, 412 |
| Winter Spice 6142872 new mod code progressing |  |  | Sk Sens 1, Aq Chr 2 | 317, 411 |

*Full text of H-Phrases can be found in Section 16.3

SECTION 4: First Aid Measures

4.1. Description of first aid measures

If inhaled:		Remove from exposure and get fresh air. Get medical attention.

If on skin:		Wash skin thoroughly with water. If irritation persist seek medical advice.

If in eyes:	Rinse with plenty of water for at least 10 minutes, keeping the eyelids open if necessary. Get medical attention.

If swallowed:	Wash out mouth with water and then drink plenty of water. Do not induce vomiting. Get medical attention.

Other first aid advice:	Not applicable.

4.2. Most important symptoms and effects, both acute and delayed

If inhaled:		Inhaling large quantities may cause intoxication.

If on skin:		May irritate skin.

If in eyes:		May irritate eyes.

If swallowed:		Swallowing large quantities may cause intoxication.

4.3. Indication of any immediate medical attention and special treatment needed

Medical treatment:	If not sure about any symptoms contact a doctor or emergency services.

SECTION 5: Firefighting Measures

5.1. Extinguishing media

Suitable extinguishing media:	Water spray, foam, dry chemical or carbon dioxide.

Unsuitable extinguishing media:	Do not use water jet.

5.2. Special hazards arising from the substance or mixture

May produce carbon dioxide, carbon monoxide and alcohol fumes.

5.3. Advice for firefighters

Wear self-contained breathing apparatus and protective clothing.

SECTION 6: Accidental Release Measures

6.1. Personal precautions, protective equipment and emergency procedures

Non-emergency personnel:	Wear appropriate protective clothing (see Section 8). Avoid contact with eyes and skin.

Emergency personnel:	Extinguish sources of ignition. Wear appropriate protective clothing (see Section 8).  Avoid contact with eyes and skin.

6.2. Environmental precautions

Prevent non-dilute material entering drains and watercourses. Advise local authorities if spillage has entered watercourses and sewer.

6.3. Methods and material for containment and cleaning up

Absorb onto inert material and transfer to suitable containers for disposal by a licensed waste contractor.

6.4. Reference to other sections

See Section 8 for protective equipment requirements.

SECTION 7: Handling and Storage

7.1. Precautions for safe handling

Handling:	Avoid contact with eyes and skin. Handle in well-ventilated area to prevent buildup of vapour. Avoid static discharge and sources of ignition.

Hygiene:	Do not eat, drink or smoke while using this product. Wash hands thoroughly after use.

7.2. Conditions for safe storage, including any incompatibilities

Store in well closed containers in a cool place, away from sources of ignition.

7.3. Specific end use(s)

Household disinfectant.

SECTION 8: Exposure Controls/Personal Protection

8.1. Control parameters

| Substance | CAS no EC/List no REACH reg. no | LTEL (8 hr TWA ppm) | LTEL (8 hr TWA mg/m3) | STEL (ppm) | STEL (mg/m3) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Ethanol 9002380 | 64-17-5 200-578-6 01-2119457610-43-xxxx | 1000 | - | - | - |  |
| Methanol (denaturant for Ethanol) 9002380 | 67-56-1 200-659-6 01-2119433307-44-xxxx | 200 | - | 250 | - |  |

8.2. Exposure controls

Engineering controls:	Provide adequate ventilation

Eye protection:		Chemical goggles or safety glasses

Face protection:		Not applicable

Hand protection:		Rubber gloves

Skin Protection:		Overalls must be worn when handling large quantities

Other protection:		None required

Respiratory protection:	If ventilation is insufficient, wear respiratory protection

Thermal hazards:		None known

Environmental controls:	See Section 6.2

SECTION 9: Physical and Chemical Properties

9.1. Information on basic chemical and physical properties

Appearance:			Clear yellow mobile liquid

Odour:				Perfumed

Odour threshold:			Not determined

PH:				Not determined

Melting/freezing point:		-114°C (Ethanol)

Initial boiling point:		78°C (Ethanol)

Boiling range:			Not determined

Flash point:			17°C

Evaporation rate:			3.4 (Ethanol)

Flammability:			Highly flammable

Decomposition temperature:	Not available

Viscosity	:			1.52 (Ethanol)

Upper explosive limit:		19% (Ethanol)

Lower explosive limit:		3.3% (Ethanol)

Vapour pressure:			5.81 (Ethanol)

Vapour density (Air=1):		1.59 (Ethanol)

Relative density:			0.85-0.87 at 20°C

Solubility:			Dispersible in water

Partition coefficient:		0.32 (Ethanol)

Auto ignition temperature:		365°C

Oxidizing properties:		Not determined

Explosive properties:		Not determined

9.2. Other information

Not available.

SECTION 10: Stability and Reactivity

10.1. Reactivity

Stable

10.2. Chemical stability

Stable

10.3. Possibility of hazardous reactions

Possible reaction with nitric acid, sulfuric acid and oxidising agents.

10.4. Conditions to avoid

Avoid high temperature, static discharge and ignition sources.

10.5. Incompatible materials

Acids, oxidising agents.

10.6. Hazardous decomposition products

None known

SECTION 11: Toxicological Information

11.1. Information on toxicological effects

Acute toxicity:			LD50 (oral rat) 6200-17800mg/kg – ethanol

LD50 (oral rat) 795mg/kg – benzalkonium chloride

Skin corrosion/irritation:		May irritate skin

Serious eye damage/irritation:	May irritate eyes

Respiratory or skin sensitisation:	Some perfumes may cause sensitisation by skin contact

Germ cell mutagenicity:		Data not available

Carcinogenicity:			Data not available

Reproductive toxicity:		Data not available

STOT-single exposure:		Data not available

STOT-repeated exposure:		Data not available

Aspiration hazard:		Data not available

11.2. Other information

N/A

SECTION 12: Ecological Information

12.1. Toxicity

LC50 (fish) 1-10mg/l 96h – benzalkonium chloride

LC50 (daphnia) <1mg/l 48h – benzalkonium chloride

12.2. Persistence and degradability

Data not available

12.3. Bioaccumulative potential

Data not available

12.4. Mobility in soil

Data not available

12.5. Results of PBT and vPvB assessment

Data not available

12.6. Other adverse effects

N/A

SECTION 13: Disposal Considerations

13.1. Waste treatment methods

Dispose of waste and residues in accordance with local authority requirements. This material must be disposed of as hazardous waste.

SECTION 14: Transport Information

14.1. UN number

1170

14.2. UN proper shipping name

Ethanol Solution (Ethyl Alcohol Solution)

14.3. Transport hazard class (es)

3

14.4. Packing group

II

14.5. Environmental hazards

Not a marine pollutant

14.6. Special precautions for user

N/A

14.7. Transport in bulk according to Annex II of MARPOL73/78 and the IBC code

N/A

Additional Information:

Product can be transported as Limited Quantity if less than 1 litre receptacle and packed in cardboard boxes up to a 30kg gross mass or shrink-wrapped trays up to a gross mass of 20kg.

SECTION 15: Regulatory Information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

Not regulated by specific legislation.

15.2. Chemical safety assessment

Not available

SECTION 16: Other Information

16.1. Reason for change

Winter morning hazards and allergens edited for new mod, cranberry and orange allergens edited for new mod.

16.2. Sources of information

Ingredient safety data sheet(s), C&L Inventory and REACH Dossiers.

16.3. Full text of H phrases from section 3

H225:	Highly flammable liquid and vapour

H301:	Toxic if swallowed

H302:	Harmful if swallowed

H304:	May be fatal if swallowed and enters airways

H311:	Toxic in contact with skin

H314:	Causes severe skin burns and eye damage

H315:	Causes skin irritation

H317:	May cause an allergic skin reaction

H318:	Causes serious eye damage

H319:	Causes serious eye irritation

H331:	Toxic if inhaled

H370:	Causes damage to organs

H400:	Very toxic to aquatic life

H410:	Very toxic to aquatic life with long lasting effects

H411:	Toxic to aquatic life with long lasting effects

H412:	Harmful to aquatic life with long lasting effects

16.4. Method of evaluation

Hazards for mixture calculated in accordance with Regulation (EC) No. 1272/2008 (GB-CLP) using the information provided for constituents (ingredients).